ANNEX 1

to the tender documents

Technical, qualitative and quantitative characteristics of the subject of procurement

(Terms of Reference)

DK 021:2015 - 71631480-8 " Services on Technical Inspection of Roads”

(Services related to obtaining an instrumental assessment of the existing condition of inspected roads)

Information on the technical, qualitative and quantitative characteristics of the subject matter of the procurement (terms of reference)

References in the tender documents to specific brands or firms, patents, designs or type of subject of the procurement, its source of origin or manufacturer should be read as - or equivalent.

1. DESIGNATION AND ABBREVIATION

The following designations and abbreviations are used in this document:

PMS - Pavement Management Information and Analytical System;

ToR - Terms of reference;

Ukravtodor - State Road Agency of Ukraine;

API (Application Programming Interface) - application programming interface;

e-road - information system Ukravtodor

GPS - global positioning system;

IRI - International roughness index;

HDM-4 - Highway Development and Management software developed by HDMGLOBAL consortium with the World Bank for modelling road construction, repair and maintenance processes

REST - Representational state transfer "transfer of the display status" - is an architectural style of interaction between components of a distributed application in the network;

XML (extensible markup language) - extensible markup language.

The service includes:

- measurement of primary data - functional, structural, geometric parameters of the pavement (Lot 1) and friction coefficient and measurements of traffic volume and stream mix (Lot 2) of roads linked to geographical coordinates;

- processing of primary data using the Contractor's software, which allows obtaining reliable information concerning the functional condition of the road;

- validation by the Contractor of the correctness of the obtained data;

- uploading to Customer’s database containing primary data concerning the physical indicators of the road, as well as functional indicators of all pavement and subgrade components with GPS-referencing (evenness, strength, bearing capacity), in formats suitable for uploading to HDM4, PMS, e-road and other systems, provided they are compatible with HDM-4 (Highway Development & Management), version 2.0. All data is transmitted in digital format.

The unit of measure of the service carried out is the set of data on the condition of the pavement layers, pavements on road sections of a particular length in line kilometers, separately by lane.

Measurement data is a series of values for each measured parameter with reference to the coordinates of the point of measurement and recording of their condition, characteristics and defects.

The scope of road condition measurement services consists of the sum of linear kilometers of all lanes of international, national, regional and some territorial roads according to the list given [13]. This volume can vary either upwards or downwards by up to 10%, with a proportional reduction in the cost of the work as a whole.

3.  Reporting requirements and measurement results

Results of work carried out are: monthly and final reports. The results of each road condition measurement must be georeferenced and must contain information about the date and time when the measuring equipment was located at the coordinates. All coordinate references shall be provided in the unified state coordinate system USK-2000 (hereinafter referred to as km+).

The software supplied by the Contractor on a free of charge basis from the beginning of the works for a period of use of two years from the date of completion of the planned works allows

- visualize and evaluate road conditions on the basis of user-defined criteria.

- calculate the necessary road quality indicators;

- interact with the Client's PMS platform and e-road via an API.

The Contractor shall provide the measuring data to the Client in a format compatible with HDM-4, together with appropriate software for such data conversion on a free-of-charge basis.

Measurement results, i.e. measurement data calculated in accordance with the current regulations and verified by the Contractor, shall be provided to the Client in a digital format compatible with the current documents of Ukraine and in HDM-4 format no later than the 5th day of the month following the reporting month. Information about the names of roads and measurement locations that occurred during the month shall be submitted to the Client in digital and paper form no later than the 5th day of the following month after the reporting month.

In the monthly report on the performance of the measurement work, the Contractor can include only those roads which parameters have been measured along the entire road length (taking into account the number of lanes in each direction), unless otherwise agreed with the Client.

The Client reserves the right to publish any information about the measurement process and results in accordance with the requirements of the Resolution of the Cabinet of Ministers of Ukraine No. 835 of 21 October 2015 "On approval of the Regulation on data sets to be made publicly available in the form of open data" (as amended). The Contractor shall not have the right to transfer to third parties and disseminate in the public domain information about the results and processes of road condition measurements.

Measurement results can be discussed each month at meetings of the public board of Ukravtodor held online or offline, with or without involvement of the Contractor's representatives.

In order to monitor the actual performance of the Contractor, all vehicles on which the measuring equipment is installed can be equipped with the Client's devices or the supplier shall provide the Client with access to appropriate software allowing the Client in a 24/7 format to obtain GPS coordinates of the vehicle routes during the measurements.

Based on the measurement results, data should be made available in three ways:

1. in the form of interim and final reports in hard copy and on a USB memory stick;

2. in the form of processed data (in HDM-4 compatible format), together with the software (hereinafter software) supplied by the Contractor on a free of charge basis with a period of free use and support of two years (from the date of transfer and signing of acceptance certificate);

3. in the form of data uploaded to the Client's e-road platform.

The transferred software should meet the following requirements:

- visualize and evaluate the condition of the road on the basis of criteria defined by the Client;

- calculate measured road condition indicators;

- Transfer data via REST API to other software.

Measurement results shall be sent digitally and in hard copy to the Client no later than the 5th day of the month following the reporting month. The names of the roads and their kilometer references for the measurements during the stage shall be transmitted to the Contractor in digital and paper form no later than the 5th day of the following month after the reporting month. In the stage report for the measurement work the Contractor can include only those roads which parameters have been measured according to the task-plan provided by the Client. In case of incomplete measurement of the road section (inconsistency with the task-plan) - the road shall not be submitted in the stage report. As a result of each phase a data set with its processing must be provided.

4. Standards and regulations

Measurement and assessment of geometry, evenness, pavement surface roughness, condition of top and bottom layers of the pavement of public roads may be carried out in accordance with:

1. DSTU 8749:2017 Road Safety. Fencing and management of road traffic in places where road works are carried out.

2. DBN B.2.3-4:2015 Highways. Part I. Design. Part II. Construction;

3. DBN B.2.3-5:2018 Streets and roads of settlements;

4. DSTU 8745:2017 Highways. Methods for measuring unevenness of the foundation and pavement

5. DSTU 8746:2017 Highways. Methods for measuring grip properties of pavement surface

6. DSTU 8747:2017 Roads. Types and lists of repair and maintenance works

7. DSTU 8954:2019 Highways. Assessment of the level of defectiveness of pavement

8. DSTU B.2.3-42:2016 Highways. Methods for determination of deformation characteristics of subgrade and pavement

9. DSTU 8824:2019 Highways. Determination of traffic intensity and composition of traffic flow

10. SOU 42.1-37641918-038:2016 "The passport of the road”

11. SOU 42.1-37641918-122:2014 "Highways. Requirements for the set of works on information support".

12. SOU 42.1-37641918-063:2016 "Geoinformation system of highways. Requirements for composition, content and application".

13. Decree of the Cabinet of Ministers of Ukraine No. 55 of 30 January 2019. "On Approval of the List of Public Roads of State Importance".

14. DSTU ISO 13473-2:2021 (ISO 13473-2:2002, IDT) Definition of pavement roughness characteristics by its surface profiles. Part 2. Terminology and basic requirements for analysis of surface roughness profiles

15. ISO 13473-3:2021 (ISO 13473-3:2002, IDT) Determination of surface pavement roughness characteristics by surface profiles. Part 3. Specification and classification of profilers

16. ISO/TS 13473-4:2021 (ISO/TS 13473-4:2008, IDT) Road surface roughness characterization by surface profiles. Part 4: Spectral analysis of pavement profiles.

17. DSTU EN 12697-8:2018 Bituminous and mineral mixtures. Methods for sampling hot asphalt mixtures. Part 8. Determination of porosity values of asphalt specimens (EN 12697-8:2003, IDT)

Measurement of parameters (distances, dimensions, thicknesses, etc.) from which the geometric and traffic parameters of the road condition can be computed can be carried out according to

- EN 13036-6 :2008 Road and airfield surface characteristics - Test methods

- ISO 13473-1: 2019 Characterization of pavement texture by use of surface profiles

- ASTM 4695 Standard Guide for General Pavement Deflection Measurements

- ASTM E1215 - 93 Standard Specification for Trailers Used for Measuring Vehicular Response to Road Roughness

- ASTM E1082-90(2017) Standard Test Method for Measurement of Vehicular Response to Traveled Surface Roughness

- ASTM D4602 - 93(2020)  Standard Guide for Nondestructive Testing of Pavements Using Cyclic-Loading Dynamic Deflection Equipment

- ASTM D6433 - 20 Standard Practice for Roads and Parking Lots Pavement Condition Index Surveys

- ASTM D4694 - 09(2020) Standard Test Method for Deflections with a Falling-Weight-Type Impulse Load Device

Lot 1: Collecting data on the functional state of the road.

1: Requirements for the organization of the measurement process and the interaction between the Client and the Contractor

Works should be carried out without blocking the traffic and limiting the speed of traffic flow on the road, which parameters are measured. When work is to be carried out directly on the road, the requirements of [1] shall be observed.

All parameters shall be measured by non-destructive methods in automated mode. The structure of the pavement is determined by means of Ground penetrating radar (GPR).

Automated mode of measurement is ensured by vehicle-mounted measuring equipment. This vehicle and its operators form a measurement team.

The measurement capacity (primary data collection) of one team under favorable weather conditions (no precipitation, dry road surface) should be at least 150 km/day due to the tight deadlines.

2.Data list

The list and format of data to be measured is given in Table 1. The measurement shall be made on the basis of a pre-approved plan of work which includes a list of roads with a code, name and kilometer reference and shall be agreed upon before the work is carried out. The task-plan shall be carried out for every single stage of work, but can be changed on the initiative of the Contractor after written agreement with the Client and identification of reasons for such changes.

Table 1

List and format of data to be collected

| Nr. | Title | Format | Note |
| --- | --- | --- | --- |
| 1 | Photographic records of the road (not less than 300 photos per km of road section) degrees with a catalogue of coordinates for each photo | *.jpg | According to the standards and regulations in section 4. |
| 2 | Materials for determining pavement defects | *.xls | According to the standards and regulations in section 4. |
| 3 | Road surface evenness of the roadway IRI | *.xls | Every 100 metres and every 1000 metres with a different file |
| 4 | Determination of pavement surface roughness (based on a 2-lane road) Number of traffic lanes - 2) | *.xls | Determination of pavement roughness parameters (based on a 2-lane road) Number of traffic lanes - 2 |
| 5 | Strength of the pavement structure | *.xls | Continuous measurement of mast modulus with frequency of data provision in number of not less than one measurement result per running meter (not less than 1000 measurement points per running kilometre) for each travel direction, in accordance with DSTU B B.2.3-42, converted into synthetic indicators in accordance with DSTU B.2.3-42 |
| 6 | Pavement structure by non-destructive methods | *.xls | Radarogram of the pavement layers and their thickness |
| 7 | Table 2 for filling the HDM-4 with the appropriate sample | *.xls | With date and time of receipt of measurements |

Reporting shall be done every month (general information on the stage of work execution) by the 5th day of the following month, indicating the scope of work executed. The indicators shall be measured according to the list in Table 2.

Provision of the service implies measurement and assessment of the actual condition of the roads listed in Table 2.

Table 2

Parameters of road condition to be measured and assessed

| Nr. | Name of group/measurement parameter/assessment | Unit of measurement |
| --- | --- | --- |
|  | Road geometry |  |
| 1 | Longitudinal slope | ‰ |
| 2 | Transverse slope | ‰ |
| 3 | Road unevenness: - longitudinal profile - rutting (rutting depth, profile deflection) | IRI, m/km mm |
|  | Road surface texture |  |
| 4 | Root Mean Square (RMS) | mm |
| 5 | Mean Profile Depth (MPD) | mm |
| 6 | Defectiveness | mlevel, m, m2, m/m sq. composite index |
|  | Assessment of the pavement layers structure |  |
| 7 | Thickness of each pavement layer ( individually) * | mm |
|  | Bearing capacity of the road |  |
| 8 | Applied load | kg |
| 9 | Area of applied load | сm2 |
| 10 | Elastic deflection | mm |
| 11 | Pavement strength of the road | МPа |
| 12 | Air temperature | °С |
| 13 | Temperature of the pavement | °С |

Note. The thickness of the pavement layer is measured to a depth of 500 mm, the thickness of the base course to a depth of 1000 mm.

Visualisation of the results of road measurement from Table 2 is made, according to Table 3.

Table 3

Means of presentation of measurement results

| Nr. | Name of group/measurement parameter/assessment | Data visualization | Format |
| --- | --- | --- | --- |
| 1 | Photo fixation | Road surface photo fixation materials (meter by meter) | *.jpg |
| 2 | Defectiveness | Materials for recording pavement defects | *.xls *.doc *.jpg |
| 3 | Evenness | Longitudinal evenness of the road | *.xls |
|  |  | Transverse evenness | *.xls |
|  |  | Longitudinal gradient | *.xls |
|  |  | Transverse gradient | *.xls |
| 4 | Assessment of the structure of the road pavement | Assessment of the pavement layer by georadar | Radarogram and top layers design of road pavements |
| 5 | Assessment of the road foundation structure | Assessment of the base of the pavement with GPR | Radarogram and design of pavement base layers. |
| 6 | Bearing capacity of the road | Value of pavement strength and all associated data. Modulus of elasticity of the pavement and safety factor, highlighting problem areas in terms of bearing capacity | *.xls |
| 7 | Structure of the pavement surface | Values by pavement surface: - - pavement roughness | *.xls |

The road condition parameters to be measured (Table 1) and their visualisation (Table 2) shall be provided by the Contractor to the Client for each linear kilometre of each individual lane of international, national, regional and some territorial roads in Ukraine, in accordance with the list given in [13]. The unit of account for the quantity of condition measurement work to be carried out is one linear kilometre of one road lane. The scope of work for the subject of the procurement is given in Table 4.

Table 4

Scope of work for the subject of the procurement

| Nr. | Name | /Notes | Quantity |
| --- | --- | --- | --- |
| 1 | IRI roadway pavement evenness ( based on 2-lane road) number of traffic lanes - 2) | According to the standards and regulations given in the section 4. | 51 000 km |
| 2 | Pavement structure strength (based on a 2-lane road, number of lanes: 2) | According to the standards and regulations given in the section 4. 1,000 measurements per lane kilometre per lane | 51 000 km |
| 3 | Determination of pavement defects (damages and deformations) on the roadway (based on a 2-lane road, the number of lanes is 2) | According to the standards and regulations given in the section 4. | 51 000 km |
| 4 | Determination of pavement surface roughness (based on a 2-lane road) Number of traffic lanes - 2) | According to the standards and regulations given in the section 4. | 51 000 km |
| 5 | Photo fixation | Every meter of pavement surface | 51 000 km |

* or relevant harmonized standards.

Lot 2: "Determination of geometric parameters of the highway, the grip properties of pavement, measuring the intensity and composition of traffic flow on it."

- Requirements for the organization of the measurement process and the interaction of the Client and the Contractor

The works must be carried out without blocking traffic and limiting the speed of traffic on the road which parameters are being measured. When carrying out work directly on the road it is necessary to comply with the requirements [1].

Measurements of all parameters are carried out by non-destructive methods in the automated mode.

- List of data

The list and format of the measured data are given in table. 1. Measurement is performed according to a pre-approved tasks schedule, which includes a list of roads with index, name and kilometer reference and agreed before the work. The tasks schedule is formed for each stage of work, but can be adjusted at the initiative of the Contractor in writing with the Client and determination of the cause of changes.

Table 1

List and format of data to be collected

|  | Name | Format | Note |
| --- | --- | --- | --- |
| 1 | Materials for determining the geometric parameters of the highway | *.xls | According to the standards and regulations given in the section 4. |
| 2 | Determination of grip properties of the road pavement surface (based on a 2-lane highway) number of lanes - 2 | *.xls | Measurement of grip qualities of the road surface (based on a 2-lane highway) number of lanes - 2 |
| 3 | Determination of the intensity and composition of the traffic flow | *.xls | Measurement of the intensity and composition of the traffic flow (based on a 2-lane highway) |

Submission of reports is carried out every month (general information on the stage of works fulfillment) until the 5th day of the following month, which indicates the amount of work performed. Measurement of indicators should be carried out according to the list in accordance with Table 2.

The provision of the service involves measuring and assessing the actual condition of the roads listed in table 2.

Table 2 Road condition parameters to be measured and evaluated

| Nr. | Name of the group / parameter of measurements / evaluation | Measurement units |
| --- | --- | --- |
|  | Road geometry |  |
| 1 | Horizontal curve radius | Radius, m |
| 2 | Vertical curve radius | Radius, m |
| 3 | Geometric parameters of the road (lane, shoulders, reinforcements, median) | mm |
| 4 | Value of the grip coefficient of the car’s wheel with the road surface |  |
| 5 | Intensity and composition of the traffic flow | pcs / hour |

Table 3

Means of presenting measurement results

| Nr. | Name of the group / parameter of measurements / evaluation | Data visualization | Format |
| --- | --- | --- | --- |
| 1 | Road geometry | Transverse profile parameters | *.xls |
| 2 | Pavement surface structure | Grip coefficient | *.xls |
| 3 | Intensity and composition of the traffic flow | Values of the number and types of vehicles | *.xls |

The parameters of the road condition measured (Table 1) and their visualization (Table 2) must be provided by the Contractor to the Client for each linear kilometer of each individual lane of international, national, regional and some territorial roads of Ukraine, in accordance with the list in [13]. The unit of calculation of the scope of works performed to measure the condition is the linear kilometer of one lane of the road. The volume of work on the subject of procurement is shown in Table 4.

Table 4

The amount of works on the subject of procurement

| Nr. | Name | Regulations | Quantity |
| --- | --- | --- | --- |
| 1 | Geometric parameters of the highway | According to the standards and regulations given in the section 4. | 51 000 km |
| 2 | Determination of grip properties of the road pavement surface (based on a 2-lane highway) number of lanes - 2 | According to the standards and regulations given in the section 4. | 51 000 km |
| 3 | Measurement of intensity and composition of the traffic flow | According to the standards and regulations given in the section 4. | 4743 measurement points |

When providing services, the Contractor shall observe environmental protection measures and comply with the requirements for safe work within the road in accordance with legal requirements.

The work shall be carried out in accordance with a works schedule approved in advance by the Client. Monitoring results shall be submitted in digital and paper form, a report on the services provided no later than the on the 6th day of the month following the reporting month.

Term of services providing: from the date of the Contract to 31 December 2022.

List of documents required by the Client to confirm compliance with the requirements of Articles 16 and 17 of the Law of Ukraine "On Public Procurement"

- DOCUMENTS FOR CONFIRMATION OF INFORMATION ON COMPLIANCE WITH QUALIFICATION CRITERIA (Article 16 of the Law)

1.1. Information certificate according to the table signed by the Head or the authorized person and sealed (if used) on the availability of the Bidder’s equipment and material and technical base necessary for the provision of services in accordance with the Terms of reference (Annex 1)

| Nr. | Name of devices, equipment, transport, etc. (indicating DSTU or EN standards, ISO and ASTM according to which measurements are made) | Quantity | Indication of affiliation * | Affiliation confirmation doc. ** |
| --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 |

* if the Bidder is the owner, indicate "own", in other cases - indicate the "right of use" (lease agreement, lease or otherwise specified by the legislation of Ukraine, which must be valid throughout the term of provision of services on the subject of procurement;

** indicate the number and date of the document provided by the Bidder as proof of affiliation in accordance with column 5 of the Table.

Mandatory minimum list and quantity of necessary machinery and equipment for the provision of services on the subject of procurement:

1. Lot 1 and Lot 2 - vehicles with installed measuring instruments that allow to measure the relevant parameters of the road and the condition of the pavement.

2. Lot 2 - means for measuring the intensity of traffic flow, portable or stationary.

3. Lot 1 and Lot 2 - Computer equipment, the technical characteristics of which allow the operation of the Contractor's software to obtain and process the results of measurements of road condition parameters and convert them into a format compatible with HDM-4

1.1.1. To confirm the availability of its own material and technical base, the Bidder must provide a balance sheet signed by the Head (authorized person) and a certified seal (if any).

In case of rent (leasing) of cars, devices, equipment, the Bidder must provide a duly certified copy of the relevant lease agreement, leasing, etc. with all annexes to it, which must be valid for the entire period of provision of services on the subject of purchase, and the original letter of confirmation or another person (party) specified in the relevant agreement, not to deny the use of his cars, devices, equipment, etc. for the provision of services by the Bidder on the subject of procurement.

1.2. The Bidder must have its own or involved test laboratory (s) / center (s) equipped with appropriate calibrated equipment for the measurements specified in Table 2 of this Bidding Document for Lot 1.

To document the presence of a test laboratory (s)

Bidder(s) / center (s) must provide: a certificate in any form of the Bidder's testing laboratory (s) / center (s) listing the ability to perform the required tests;

Testing laboratories may be mobile and located in vehicles owned by the Contractor or a third party. Requirements for its involvement in the performance of measurement work see above.

Documents required to confirm information on the absence of grounds for refusal to participate in the procurement procedure on the basis of Article 17 of the Law "On Public Procurement"

| Grounds for rejection | Bidders in the procurement procedure |  |  |  |  | Winner of the procurement procedure |
| --- | --- | --- | --- | --- | --- | --- |
| Part one of Article 17 of the Law | Method of verifying information | Document name (Name of verification method) | Requirements for the document (Description of the method of confirmation) |  | Confirmation form |  |
| 1) The Client has indisputable evidence that the Bidder of the procurement procedure offers, gives or agrees to give directly or indirectly to any official (official) of the Client, other state body remuneration in any form, a proposal for employment, a valuable thing, service, etc.) in order to influence the decision to determine the winner of the procurement procedure or the application by the Client of a particular procurement procedure | The Client checks the information independently | - | - |  | - | checked directly by the Client during the procurement procedures, documents from the winner are not required |
| 2) information on a legal entity that is a Bidder in the procurement procedure is entered into the Unified state register of persons who have committed corruption or corruption-related offenses | Is checked directly by the Client in the Unified state register of persons who have committed corruption or corruption-related offenses. Documents, information from the Bidders are not required | - | - |  | - | Is checked directly by the Client in the Unified state register of persons who have committed corruption or corruption-related offenses, documents from the winner are not required |
| 3) an official (servant) of the Bidder who is authorized by the Bidder to represent his interests during the procurement procedure, a natural person who is a Bidder, was prosecuted in accordance with the law for committing a corruption offense or an offense related to corruption | Is checked directly by the Client in the Unified state register of persons who have committed corruption or corruption-related offenses. Documents, information from the Bidders are not required | - | - |  | - | is checked directly by the Client in the Unified state register of persons who have committed corruption or corruption-related offenses, documents from the winner are not required |
| 4) the business entity (Bidder) during the last three years was prosecuted for violation envisaged by paragraph 4 of the second part of Article 6, paragraph 1 of Article 50 of the Law of Ukraine "On Protection of Economic Competition", in the form of anticompetitive concerted actions tender results | is checked directly by the Client in the Consolidated information on the decisions of the Committee bodies on recognition of violations of the legislation on protection of economic competition in the form of distortion of the results of tenders and imposition of fines, documents from Bidders are not required | - | - |  | - | is checked directly by the Client in the Consolidated information on the decisions of the Committee bodies on recognition of violations of the legislation on protection of economic competition by economic entities in the form of distortion of results of tenders and imposition of a fine, documents from the winner are not required |
| 5) a natural person who is a Bidder in the procurement procedure has been convicted of a criminal offense committed for selfish motives (in particular, related to bribery and money laundering), whose criminal record has not been removed or expunged in the manner prescribed by law | It is checked according to the information provided by the Bidder | Confirmation when submitting a tender offer or filling in the electronic field "Application", or providing a certificate in any form as part of the tender offer | Information in any form that a natural person who is a Bidder in the procurement procedure has not been convicted of a criminal offense committed for selfish motives (in particular, related to bribery and money laundering), whose criminal record has not been removed or expunged in established by law, by confirming when submitting a tender offer or filling in the electronic field "Application", or providing a certificate in any form in accordance with the letter of the Ministry of Economy ref. № 3304-04 / 34835-06 dated 03.06.2020 |  | Application | certificate (complete), issued by the Ministry of Internal Affairs of Ukraine, with information that the individual who is the winner (un) prosecuted, no (presence) of a criminal record or restrictions under criminal procedural law of Ukraine. The document must be as of the date not earlier than the date of publication of the notice of intent to enter into a procurement contract in the electronic procurement system. |
| 6) an official (official) of the Bidder who signed the tender offer (or authorized to sign the contract in the case of a negotiated procurement procedure), was convicted of a criminal offense committed for selfish motives (in particular, related to bribery, fraud and embezzlement) funds), the conviction of which is not removed or not repaid in the manner prescribed by law |  |  | Information in any form that the official (servant) of the Bidder, who signed the tender offer, was not convicted of a criminal offense committed for selfish motives (including bribery, fraud and money laundering), a criminal record which is not withdrawn or repaid in the manner prescribed by law, by confirming when submitting a tender or filling in the electronic field "Application", or providing a certificate in any form in any form in accordance with the letter of the Ministry of Economy ref. № 3304-04 / 34835-06 dated 03.06.2020 |  | Application | certificate (complete), issued by the Ministry of Internal Affairs of Ukraine, with the information that the official (servant) of the winner, who signed the tender offer, (not) prosecuted, no (presence) of a criminal record or restrictions provided by the criminal procedure legislation of Ukraine . The document must be as of the date, not earlier than the date of publication of the notice of intent to enter into a procurement contract in the electronic procurement system. |
| 7) the tender offer submitted by the Bidder of the competitive procurement procedure or participation in the negotiated procedure is taken by the Bidder who is related to other Bidders of the procurement procedure and / or to the authorized person (persons) and / or to the Head of the Client | is checked directly by the Client during the procurement procedures, in particular information on the list of founders (Bidders) of the legal entity; last name, first name, patronymic of persons elected (appointed) to the governing body of a legal entity can be viewed in the Unified state register of legal entities, natural persons - entrepreneurs and public formations | - | - |  | - | Is checked directly by the Client during the procurement procedures, documents from the winner are not required |
| 8) The Bidderof the procurement procedure has been declared bankrupt in accordance with the procedure established by law and liquidation proceedings have been opened against him | is checked directly by the Client in the Unified register of enterprises in respect of which bankruptcy proceedings have been initiated, documents from the Bidders are not required | - | - |  | - | is checked directly by the Client in the Unified register of enterprises in respect of which bankruptcy proceedings have been initiated, documents from the winner are not required |
| 9) in the Unified state register of legal entities, individual entrepreneurs and public associations there is no information provided for in paragraph 9 of the second part of Article 9 of the Law of Ukraine "On State Registration of Legal Entities, Individual Entrepreneurs and Public Associations" (except non-residents) | Is checked directly by the Client in the Unified state register of legal elntities, individuals - entrepreneurs and public associations, documents from the Bidders are not required | - | - |  | - | Is checked directly by the Client in the Unified state register of legal elntities, individuals - entrepreneurs and public associations, documents from the winner are not required |
| 10) a legal person that is a Bidder in the procurement procedure (except for non-residents) does not have an anti-corruption program or an anti-corruption program commissioner if the cost of purchasing goods (goods), services (services) or works is equal to or exceeds UAH 20 million | The Client does not require documents, information confirming the absence of grounds. Or if the purchase price exceeds UAH 20 million - it is checked according to the information provided by the Bidder | - or if the purchase cost exceeds UAH 20 million - confirmation when submitting a tender offer or filling in the electronic field "Application", or providing a certificate in any form as part of the offer |  | - or if the cost of procurement exceeds UAH 20 million - information in any form that the legal entity that is a Bidder in the procurement procedure (except non-residents) has an anti-corruption program and the Commissioner for Anti-Corruption Program by confirming when submitting a tender or filling in an electronic field "Application", and providing an anti-corruption program and a copy of the order appointing the Commissioner for the implementation of the anti-corruption program. If the Bidder in the procurement procedure is an association of Bidders, the Anti-corruption program and a copy of the order appointing the Commissioner for the implementation of the Anti-corruption program shall be provided separately for each of the Bidders of such association. | Application | documents from the winner are not required |
| 11) A Bidder in a procurement procedure is a person to whom a sanction has been applied in the form of a ban on public procurement of goods, works and services in accordance with the Law of Ukraine "On Sanctions" | is checked directly by the Client, documents from Bidders are not required (according to all current decisions of the National Security and Defense Council on application of personal special economic and other restrictive measures (sanctions) ", approved by Decrees of the President of Ukraine |  |  |  |  | checked directly by the Client during the procurement procedures, documents from the winner are not required |
| 12) an official (servant) of the Bidder of the procurement procedure authorized by the Bidder to represent his interests during the procurement procedure, a natural person who is a Bidder has been prosecuted in accordance with the law for committing an offense related to the use of child labor or any -what forms of human trafficking | It is checked according to the information provided by the Bidder | Confirmation when submitting a tender offer or filling in the electronic field "Application", or providing a certificate in any form as part of the proposal | Information in any form that the official (official) of the Bidder of the procurement procedure, authorized by the Bidder to represent his interests during the procurement procedure, the natural person who is the Bidder, was not prosecuted under the law for committing an offense, related to the use of child labor or any form of trafficking in human beings, by confirming when submitting a tender offer or filling in the electronic field "Application", or providing a certificate in any form in accordance with the letter of the Ministry of Economy. № 3304-04 / 34835-06 dated 03.06.2020 |  | Application | provided by the winner in any form |
| 13) The Bidder of the procurement procedure has arrears of taxes and fees (mandatory payments), unless such Bidder has taken measures to install and defer such arrears in the manner and under the conditions specified by the legislation of the country of registration of such Bidder. Measures for installment and deferral of such debt in the manner and under the conditions specified by the legislation of the country of registration of such Bidder | It is checked according to the information provided by the Bidder | Confirmation when submitting a tender offer or filling in the electronic field "Application", or providing a certificate in any form as part of the proposal | Information in any form that the Bidder has no arrears of taxes and fees (mandatory payments) by confirming when submitting a tender or filling in the electronic field "Application", or providing a certificate in any form in accordance with the proposal in accordance with letter of the Ministry of Economy vyh. № 3304-04 / 34835-06 dated 03.06.2020. If a Bidder has arrears of taxes and fees (mandatory payments), it shall provide information in any form that such Bidder has taken measures to install and defer such arrears in the manner and under the conditions specified by the legislation of the country of registration of such Bidder, by confirming when submitting a tender offer or filling in the electronic field "Application", or providing a certificate in any form in the proposal in any form in accordance with the letter of the Ministry of Economy ref. № 3304-04 / 34835-06 dated 03.06.2020 |  | Application | Certificate issued by the State Tax Service of Ukraine on the absence of arrears of payments, control over the collection of which is entrusted to the supervisory authorities, the form of which is approved by the order of the Ministry of Finance of Ukraine dated 03.09.2018 №733. It is automatically formed in the electronic procurement system as a result of integration of the electronic procurement system with the information systems of the State Fiscal Service of Ukraine. Adoption of the relevant decision of the body of revenues and fees and the conclusion of an agreement on installments (deferrals) in accordance with the Order of the Ministry of Revenues and Duties of Ukraine dated 10.10.2013 № 574 |
| Part two of Article 17 of the Law |  |  |  |  |  |  |
| The bidder has not fulfilled its obligations under the previously concluded procurement contract with the same Client, which led to its early termination, and sanctions in the form of fines and / or damages - within three years from the date of early termination of such contract. A Bidder in a procurement procedure who is in the circumstances specified in part two of Article 17 of the Law may provide confirmation of the measures taken to prove its reliability, despite the existence of appropriate grounds for refusing to participate in the procurement procedure. | It is checked according to the information provided by the Bidder | Upload by the Bidder of information in the form of a certificate in any form during the submission of the tender offer through the electronic procurement system | The Bidder of the procurement procedure in the electronic procurement system during the submission of the tender offer confirms the absence of grounds (part 3 of Article 17 of the Law). Information is provided in the form of a certificate in any form in accordance with the letter of the Ministry of Economy vyh. 3304-04 / 34835-06 dated 03.06.2020 |  | Information in the form of a certificate in any form | provided by the winner in any form |

Signature of the Head or authorized person of the Bidder - legal entity, natural person - entrepreneur, certified by the seal (in case of its use).

ІІ. OTHER documents provided by the Bidder as part of the tender offer:

The certificate, drawn up in any form signed by the authorized person of the Bidder and containing the information of the Bidder, namely:

a) details (location (residence), telephone, fax, contact phone);

b) the management and / or the authorized person (s) of the Bidder to sign the proposal and the procurement contract (position, surname, name, patronymic);

c) information on the details of the bank account on which the payment will be made under the agreement.

- Documentary confirmation of the authority to sign the tender offer documents and the procurement contract, namely: scanned copy (original or copy of the document) of the protocol decision of the Bidders (shareholders, owners) and the order on appointment and / or power of attorney (or other document) to the authorized ) the person (persons) of the Bidder (duly executed taking into account the statutory documents);

- information certificate with a description of environmental protection measures during the provision of services according to the subject of procurement.

In order to fulfill the requirements for submission of documents provided for in Annex 2 to the tender documentation, non-resident bidders shall submit as part of their tender offer the documents provided for by the legislation of the countries where they are registered with a translation into Ukrainian.

If the Bidder in accordance with current legislation or a non-resident Bidder in accordance with the legislation of the country of registration is not required to draw up any of these documents, such Bidder provides an explanatory letter in any form, signed by the authorized person of the Bidde and certified with a seal (if any) in which indicates the legal grounds for failure to provide the above documents. Documents may be submitted in duplicate in a foreign language and with a translation into Ukrainian. The Ukrainian version of the translation has the highest force and must be pre-agreed to avoid inconsistencies with the requirements of the tender and the legislation of Ukraine.